Molecular dynamics investigation of the effect of interlayer cavities of the structure of calcium silicate hydrate at the atomistic scale

# Davoud Tavakoli¹, Mohammad Hajmohammadian Baghban²,*

1 Department of Civil Engineering, Shahrekord Branch, Islamic Azad University, Shahrekord, Iran, d.tavakoli@sru.ac.ir

2 Department of Manufacturing and Civil Engineering, Norwegian University of Science and Technology (NTNU), 2815 Gjøvik, Norway, mohammad.baghban@ntnu.no

# Key Words: Calcium silicate hydrate, Molecular Dynamics method, concrete, cavity.

Calcium silicate hydrate (CSH) gel, as the most important component of hydration products, has the most significant effect on the properties of hardened cement paste [1, 2]. One of the most critical factors affecting the mechanical properties of CSH is the interlayer cavities in the gel [3]. In this study, the effect of these cavities on Young's modulus of CSH has been investigated. For this purpose, first, the atomic structure of CSH is created, and then interlayer cavities with different dimensions are created inside the structure.  For modeling, first, a super cell with dimensions of 3 × 6 × 1 times the unit cell of Tobermorite is prepared, and then each of these layers are placed on both sides of the new cell, and a space is created between these two layers. This distance is basically the cavity between the layers. The cavities are made in three sizes of 0.5, 1, and 2 nm, and water is then absorbed by the Grand Canonical Monte Carlo (GCMC) method. Then the created structures are simulated using the molecular dynamics method, and finally, Young's modulus is extracted. The results confirm the effect of the interlayer cavity on reducing the Young's modulus of CSH gel. As the dimensions of the cavity increase, the amount of water absorbed in the cavity increases, leading to a decrease in the Young's modulus of CSH.

REFERENCES

- A. Tarighat, and D. Tavakoli, Estimation of the elastic properties of important calcium silicate hydrates in nano scale-a molecular dynamics approach. Journal of Rehabilitation in Civil Engineering, Vol. 7, No. 4, pp. 18-36, 2019.

- D. Tavakoli, P. Gao, A. Tarighat  and G. Ye, Multi-scale approach from atomistic to macro for simulation of the elastic properties of cement paste. Iranian Journal of Science and Technology, Transactions of Civil Engineering, Vol. 44, No. 3, pp. 861-873, 2020.

- D. Tavakoli, A. Tarighat and J. Beheshtian, Nanoscale investigation of the influence of water on the elastic properties of C–S–H gel by molecular simulation. Proceedings of the Institution of Mechanical Engineers, Part L: Journal of Materials: Design and Applications, Vol. 233, No. 7, pp. 1295-1306, 2019.